STANDARD MICROSYSTEMS CORPORATION

Novell NetWare 386 v4.x Server Driver for

SMC EtherPower II 10/100 Ethernet/Fast Ethernet PCI Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1998 SMC Networks, Inc.

All rights reserved.

This installation guide describes:

o  An overview.

o  Preinstallation hardware and software requirements.

o  Supplied files.

o  Adapter installation notes.

o  How to use EZStart to copy the Novell NetWare 386 v4.x server

driver (SMCPWR2.LAN) from the SuperDisk to your server.

o  How to load the LAN driver and establish network connection.

o  Driver command line parameters.

o  Installing multiple adapters.

o  Troubleshooting.

o  Error messages.

NOTE: This adapter supports Auto Negotiation of line speed between the

adapter and the hub.

OVERVIEW

This file explains how to install and load the Novell NetWare 386

v4.x server driver.

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with 80486 or higher

microprocessor and an available PCI slot.

- An EtherPower II 10/100 PCI adapter.

Software Requirements:

- Novell NetWare 386 v4.x

SUPPLIED FILES

General files

o  INSTALL.DOC      This file.

o  RELEASE.DOC      Release notes for this version of the driver.

Files to install the Novell NetWare 386 v4.x driver

o  ETHERTSM.NLM

o  MSM.NLM

o  SAMPLE.4X

o  SMCPWR2.LAN

o  SMCPWR2.LDI

NOTE: NLM files are copyrighted by Novell. Do not distribute these

files without permission from Novell.

ADAPTER INSTALLATION NOTES

Refer to the EtherPower II 10/100 user guide for

complete installation instructions.

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy all

the files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

LOADING THE DRIVER

Installing the LAN driver is part of the NetWare installation

process. Use the following instructions with those

in the NetWare 386 installation manual to install the driver.

You will need the SuperDisk during this procedure.

1. Refer to the "Load LAN driver module" section in the Novell

NetWare 386 v4.x installation manual for instructions on

installing NetWare.

2. Follow the instructions on starting NetWare and configuring

the server. Use the Load command to load the LAN driver.

If the LAN driver is not located in the default directory,

specify the path to the file on the command line. The

Load command will prompt you for configuration parameters

if they are not supplied on the command line.

The Load command line parameters may be saved in an

AUTOEXEC.NCF file, so they execute automatically every time

the server starts. Refer to the NetWare 386 installation

manual on how to create an AUTOEXEC.NCF file.

Note: the following steps are specific to loading SMCPWR2.LAN.  For

examples, please refer to the SAMPLE.4X demo file on this diskette.

3. To load the driver, type the following at the Netware System Console:

"LOAD SMCPWR2 SLOT=YY"

where yy is the PCI logical slot number where the SMC EtherPower II 10/100

adapter is installed.

or with NetWare 386 v4.11:

"LOAD SMCPWR2"

to cause the Netware Server to indicate the SMC EtherPower II 10/100

adapters found & their logical slot numbers.  You may then choose the

adapter you want to load.

Note, that there may be no correspondence between PCI logical slot numbers

and PCI physical slot numbers.

4. At this point, a protocol must be bound to the driver. For example:

BIND IPX TO SMCPWR2 NET=XX

where xx is the logical network address for the frame type

indicated in your load command.

DRIVER COMMAND LINE PARAMETERS

The only required parameter is the slot number.

Example: load C:SMCPWR2.LAN SLOT=yy<Enter>

where yy is the PCI logical slot number where the SMC EtherPower 10/100B

adapter is installed.

PCI logical slots are counted from 16 and up.

There are no required parameters in NetWare 4.11.

The following is a list of optional command line parameters:

MediaType=AutoDetect

=UTP

=BNC

- Selects the cable type.  When AutoDetect is selected, the

active media is determined by the driver.  If the active

media is UTP, auto-negotiation will be used to determine

the best available line speed and duplex mode.

(The LINESPEED keyword is only valid when MediaType=UTP is used.)

LINESPEED=AUTONEGOTIATE

=10

=100

=10FULLDUPLEX

=100FULLDUPLEX

- Selects line speed to be x Mbps (x = 10 or 100), along

with the optional full duplex mode.

When AUTONEGOTIATE is selected, the best available line

speed and duplex mode is determined automatically.

TXBUFFERS=t   - Allocates t transmit buffers (t is between 1 and 10).

Default is 3. 3 should be optimal in most environments.

RXBUFFERS=r   - Preallocates r receive buffers (r is between 1 and 40).

Default is 10. In high traffic and bursty environments,

increasing r may improve performance.

Note that the adapter is preprogrammed with Auto Negotiate on the UTP port

as the default configuration. The preprogrammed configuration can be

changed by running EZSTART. Some of the command line parameters mentioned

above are used to override the default configuration. In the absence of

such parameters, the default configuration will be used by the Netware

driver.

INSTALLING MULTIPLE ADAPTERS

If there is more than one EtherPower II 10/100 adapter, the driver will detect

them and ask you for a logical slot selection, or you can specify the

logical slot number in the LOAD command line.

For examples, please refer to the SAMPLE.4X demo file on this

diskette.

NOVELL NetWare 386 v4.x SERVER DRIVER ERROR MESSAGES

"Adapter hardware failure detected (initialization)"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test

process due to a hardware problem. Check the adapter

for configuration conflicts.

"Other system ROM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

ROM in the system. Reconfigure the adapter to use a

different RAM address and retry the installation.

"Cannot allocate host memory"

The driver is unable to allocate system memory for the adapter

because the system is out of memory.

"Cannot allocate the required number of RCBs"

The driver cannot allocate the number of RCBs specified in the

RX_BUFFERS command line parameter. Decrease the value of the

RX_BUFFERS parameter or configure more RCBs for the system.

Refer to Netware documentation on how to configure more RCBs.

NOVELL NetWare 386 v4.x SERVER DRIVER WARNING MESSAGES

"Unneeded Memory Address ignored"

You specified an unnecessary memory address parameter when

loading the driver. A software-specified memory address will

be used instead.

"Unneeded Memory Length ignored"

You specified an unnecessary memory length parameter when

loading the driver. A software-specified memory address will

be used instead.

"Unneeded Interrupt number ignored"

You specified an unnecessary interrupt number parameter when

loading the driver. A software-specified interrupt number will

be used instead.

NOVELL NetWare 386 v4.x SERVER DRIVER CUSTOM COUNTERS

You can use the MONITOR.NLM server utility to view these counters.

TxOKSingleCollision

The number of packets transmitted successfully that

encountered only one collision during transmission. It is

normal for this counter to increment on busy networks.

TxOKMultipleCollisions

The number of packets transmitted successfully that

encountered more than one collision during transmission.

It is normal for this counter to increment on busy networks.

TxOKButDeferred

The number of successfully transmitted packets that were

delayed because the medium was busy. This counter may

increment on a busy network.

TxAbortLateCollision

The number of collisions detected after the packet started

transmitting. The packet was not sent successfully due to

a collision during transmit. It is normal for this counter

to increment occasionally due to packet collisions. If

this counter increments frequently, it may indicate a

problem with another NIC adapter on the network.

TxAbortExcessCollisions

The number of packets that were not transmitted because the

maximum collision limit was exceeded (16 collisions). This

counter may occur occasionally if there is high network

traffic.

TxAbortCarrierSense

This counter indicates the number of times the carrier-sense

signal was not-asserted or was de-asserted during transmission

of a packet without a collision. This counter may increment

occasionally during periods of high network traffic. If this

counter increments frequently, it may indicate that there is

a cabling problem or a problem with another LAN adapter on

the network.

RxAbortFrameAlignment

The counter indicates the number of packets that were rejected

to an alignment error. The length of the packet did not pass

the Frame Check Sequence (FCS) check. This counter may incre-

ment occasionally due to collisions occurring on a busy net-

work. If the counter increments frequently it may indicate a

problem with another LAN adapter on the network.

AdapterResetCount

The number of times the NIC was reset due to a transmit

time-out error or an unrecoverable error. This counter may

increment occasionally during normal network operation.

If this counter increments frequently, it may indicate that

there is a problem with the LAN adapter.

ReceiveLostPacketCount

This count contains the total number of packets that were

lost due to an overrun of receive buffers. This counter

may increment occasionally during periods of high network

traffic. If this counter increments frequently, you can

reload the driver with a higher RX_BUFFERS parameter value.

TransmitFIFOUnderrunCount

This counter indicates the total number of times that the

NIC could not access the bus before the FIFO buffer emptied.

Transmission of the packet is aborted when this event

happens. This counter may occasionally increment on a busy

network. If this counter increments frequently, it may

indicate a problem with the LAN adapter.

TransmitTimeoutCount

The number of times a transmit complete interrupt did not

occur within 2 seconds of starting a packet transmission.

When this occurs, the NIC is reset and an attempt is made

to re-transmit the packet. This counter may occasionally

increment on a busy network. If this counter increments

frequently, it may indicate a problem with the network

cabling or the LAN adapter.

AdapterMiscErrorCount

This counter indicates a nonfatal error occurred. This

counter may increment occasionally during normal operation.

If this counter increments frequently, it may indicate a

problem with the LAN adapter.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

EtherPower II 10/100, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation. Other products and company names are

registered trademarks or trademarks of their respective companies.

#